#include "I2Cdev.h"

#include <PID_v1.h>

#include "MPU6050_6Axis_MotionApps20.h"

MPU6050 mpu;

bool dmpReady = false;

uint8_t mpuIntStatus;

uint8_t devStatus;

uint16_t packetSize;

uint16_t fifoCount;

uint8_t fifoBuffer[64];

Quaternion q;

VectorFloat gravity;

float ypr[3];

double setpoint = 182;

double Kp = 15;

double Kd = 0.9;

double Ki = 140;

double input, output;

PID pid(&input, &output, &setpoint, Kp, Ki, Kd, DIRECT);

volatile bool mpuInterrupt = false;

void dmpDataReady() {

mpuInterrupt = true;

}

void setup() {

Serial.begin(115200);

mpu.initialize();

mpu.testConnection();

devStatus = mpu.dmpInitialize();

mpu.setXGyroOffset(-479);

mpu.setYGyroOffset(84);

mpu.setZGyroOffset(15);

mpu.setZAccelOffset(1638);

if (devStatus == 0) {

mpu.setDMPEnabled(true);

attachInterrupt(0, dmpDataReady, RISING);

mpuIntStatus = mpu.getIntStatus();

dmpReady = true;

packetSize = mpu.dmpGetFIFOPacketSize();

pid.SetMode(AUTOMATIC);

pid.SetSampleTime(10);

pid.SetOutputLimits(-255, 255);

}

pinMode(6, OUTPUT);

pinMode(9, OUTPUT);

pinMode(10, OUTPUT);

pinMode(11, OUTPUT);

analogWrite(6, LOW);

analogWrite(9, LOW);

analogWrite(10, LOW);

analogWrite(11, LOW);

}

void loop() {

if (!dmpReady) return;

while (!mpuInterrupt && fifoCount < packetSize) {

pid.Compute();

Serial.print(input);

Serial.print(" =>");

Serial.println(output);

if (input > 150 && input < 200) {

if (output > 0)

Forward();

else if (output < 0)

Reverse();

} else {

Stop();

}

}

mpuInterrupt = false;

mpuIntStatus = mpu.getIntStatus();

fifoCount = mpu.getFIFOCount();

if ((mpuIntStatus & 0x10) || fifoCount == 1024) {

mpu.resetFIFO();

Serial.println("FIFO overflow!");

} else if (mpuIntStatus & 0x02) {

while (fifoCount < packetSize) fifoCount = mpu.getFIFOCount();

mpu.getFIFOBytes(fifoBuffer, packetSize);

fifoCount -= packetSize;

mpu.dmpGetQuaternion(&q, fifoBuffer);

mpu.dmpGetGravity(&gravity, &q);

mpu.dmpGetYawPitchRoll(ypr, &q, &gravity);

input = ypr[1] * 180 / M_PI + 180;

}

}

void Forward() {

analogWrite(6, output);

analogWrite(9, 0);

analogWrite(10, output);

analogWrite(11, 0);

Serial.print("F");

}

void Reverse() {

analogWrite(6, 0);

analogWrite(9, output * -1);

analogWrite(10, 0);

analogWrite(11, output * -1);

Serial.print("R");

}

void Stop() {

analogWrite(6, 0);

analogWrite(9, 0);

analogWrite(10, 0);

analogWrite(11, 0);

Serial.print("S");

}